# DIY Classroom Aeroponics Tower Plans

These plans describe a simple vertical aeroponics tower suitable for a small classroom (grades 4–8). The design fits through a standard doorway and runs on a single standard electrical outlet.

## Tower Specifications

- Height: 5–6 feet

- Footprint: 18–24 inch diameter

- Sites: 16–24 planting sites

- Reservoir volume: 15–20 gallons

- Recommended plants: leafy greens, basil, mint, dwarf cherry tomatoes, strawberries

## Main Components

- Vertical tower body (stacked PVC sections or modular food-safe stackers)

- Enclosed root chamber inside the tower (PVC Pipe works well for this, but you can be creative here)

- 15–20 gallon reservoir with lid

- Submersible pump rated for 80–120 psi using an external pressure pump or high-pressure pump kit

- 8–16 aeroponic misting nozzles (50–140 micron droplet size)

- Digital cycle timer (seconds resolution)

- Flexible high-pressure tubing and fittings

- LED grow light ring or 4-bar fixture on a simple frame

I created an Amazon list with most of what you need here: https://amzn.to/4bJPmr1

## Operating Targets

- pH: 5.5–6.5

- EC: 1.0–1.8 mS/cm for leafy greens and herbs

- Mist cycle (starter setting): 5–10 seconds ON, 3–5 minutes OFF

- Water temperature: 65–72 °F

Classroom Notes

- Design leaves one side visually open so students can see roots.

- All electrical components mount above the reservoir line.

- Include a laminated safety checklist and student job chart next to the tower.